A. MICROPROCESSORS

1. Give a diagram of a computer organization and explain the function of each block.

- Input: Devices like keyboard and mouse used to give data to the computer.

- Processing: Where data are processed into meaningful information; includes temporary storage (RAM) for data currently being processed.

- Output: Devices like monitors (softcopy) and printers (hardcopy) to show results.

- Permanent storage: Devices like hard disks, floppy drives, and CDROM drives to store data permanently for later use.

2. With a help of diagram, explain the basic three components of a processor.

- Control Unit: Instructs the rest of the computer on how to follow program instructions; directs data movement and control signals between the CPU and external devices.

- Arithmetic/Logic Unit (ALU): Performs arithmetic (addition, subtraction, multiplication, division) and logical operations (AND, OR, XOR) for comparisons and decisions.

- Registers: Used by the processor for temporarily store data being processed.

3. How much RAM size would a 64 bits processor support?

- A 64bits processor supports 264 bytes (approximately 18,400,000,000,000,000,000 bytes) of memory.

4. Explain the factors influencing the performance of the processor.

- Speed/rate (measured in GHz)

- Number of bits (32bit or 64bit)

- Amount of cache memory (L1, L2, L3)

5. The speed of the processor is measured in gigahertz (GHz) while the data transfer rate is measured in bytes per second (MB/s).

B. MEMORIES

1. What is mean by primary memory, secondary memory and mass storage devices?

- Primary memory (e.g., RAM) is directly connected to the CPU.

- Secondary memory (e.g., hard disks) is not connected directly to the CPU due to lower speed.

- Mass storage devices (e.g., magnetic and optical memories) are used to store high volumes of data.

2. Define the terms storage capacity, memory word, storage location, and word length.

- Storage capacity: Total number of bits a memory device can store.

- Memory word: A group of bits accessed as a fixed size.

- Storage location: A unique address in memory where a word is stored.

- Word length: The number of bits in a group of data (word).

3. Define the following terms: (i) Access Time (ii) Cycle time (iii) Bandwidth (iv) Volatile memories

- Access Time: Average time required to read a fixed amount of information from a selected memory location.

- Cycle time: Minimum time that must elapse between the initiation of two consecutive memory accesses tC = tA + tD E L AY

- Bandwidth: Maximum amount of information transferred per second (Bandwidth = 1/ tC )

- Volatile memories: Memories where stored information vanishes when power is switched off.

4. Explain organization of tracks in magnetic memories with the help of figure.

- The media is divided into concentric circles called tracks. Each track is further divided into a number of sectors where data blocks are stored.

5. What are optical memories?

- Optical memories are storage devices (e.g., CDROM, CDR, CDRW, DVD) that use lasers to read and write data on a reflective disk surface using pits and lands.

6. Explain basic semiconductor memory organization with the help of figure.

- Basic semiconductor memory organization consists of a storage cell array, read/write control circuit, address decoder connected to a memory address register, and a buffer register for data transfer.

7. With the help of figure explain the memory cell addressing.

- Memory cells are organized in a rectangular array of m rows and n columns. To select a particular cell, the appropriate row and column number (the address) are specified.

8. Explain the memory cell organization. How a particular cell can be selected in this case?

- Cells are organized as an array of rows and columns. A specific cell is selected by activating its unique row and column simultaneously. The intersection of the activated row and column is the selected cell.

9. A memory has access time of 50 n sec. It has an additional delay called precharge time of 20 n sec. Find out the access rate and bandwidth of this unit?

- Access rate = 1 / tA = 1 / 50 nsec = 20,000,000 words/sec.

- Bandwidth = 1 / tC = 1 / (50 nsec + 20 nsec) = 1 / 70 nsec ≈ 14,285,714 words/sec.

10. What is Dynamic RAM? Why are they most widely used?

- Dynamic RAM (DRAM) is a read/write memory that uses a capacitor to store 1bit. It is widely used because it offers greater storage capacity than SRAM and is less expensive.

11. What is Static RAM? Why are they most widely used?

- Answer: Static RAM (SRAM) holds stored data indefinitely if power is on. It is widely used for cache memory because it is very fast.

12. Explain how a hard disk reads information it stores.

- The read/write head senses the state of magnetization on the disk surface. Movement of the disk changes the magnetic field, inducing a voltage in the head coil. The polarity of the induced voltage gives the stored binary bit.

13. Provide advantages and disadvantages of a hard disk as a storage media.

Advantages:

- Largescale data storage for long time,

- Very low cost per bit.

Disadvantages:

- Slower speed (seek and latency time),

- Moving mechanical parts make it unreliable.

14. Explain how the CDROM stores data.

- Data is stored as indentations (pits) and bumps (lands) on a reflective surface. A laser beam is aimed at the disk; pits reflect less light (read as 0s), and lands reflect more light (read as 1s).

15. Provide advantages and disadvantages of a CDROM as a storage media.

Advantages:

- Huge amount of data storage,

- Low cost, easily transferable,

- Reliable (few mechanical parts).

Disadvantages:

- Complexity of the read/write head.

C. SYSTEM BUSES AND BASIC CARDS

1. Define a computer system bus.

- A computer system bus is a parallel collection of conductors that carry data and control signals from one component to another.

2. Explain the types of system buses based on the type of information they carry.

- Address bus: Carries addresses, unidirectional.

- Data bus: Carries data, bidirectional.

- Control bus: Carries control/timing signals.

3. What is the difference between internal and external buses?

- The internal bus (System Bus) connects components inside the case (CPU, memory). The external bus (Expansion Bus) connects external devices, peripherals, and expansion slots.

4. What do you understand by expansion bus?

- Expansion bus is the external bus that allows for the expansion of the computer's capabilities by connecting external devices and expansion cards.

5. With a block diagram explain the system bus related to computer operation.

- A diagram showing the CPU connected via an address bus, data bus, and control bus to memory and I/O elements. (See `media/image16.png` in the notes).

6. Explain various expansion slots available on a computer motherboard.

- ISA (Industry Standard Architecture)

- EISA (Enhanced ISA)

- PCI (Peripheral Component Interconnect)

- AGP (Accelerated Graphic Port)

7. Give the difference between ISA and EISA expansion slots.

- ISA is a 16bit slot at 8 MHz. EISA is a 32bit slot that supports bus mastering and is configured with software, but still has an 8 MHz bus speed.

8. Differentiate ISA from PCI expansion slots.

- ISA is a 16bit, 8 MHz slot. PCI is a 32bit local bus slot that talks to the motherboard at 33 MHz, offering significant improvement.

9. What do you understand by interrupt request lines in a computer?

- Interrupt Request lines (IRQs) are special pathways directly to the processor that a device uses to get the CPU's attention when it needs to communicate.

10. Explain the function of DMA channels and I/O addresses.

- DMA channels: Allow specific devices to access memory directly without passing through the CPU for faster transfers.

- I/O addresses: The starting addresses in memory assigned to a device for exchanging data.

11. Explain the term "Bus Mastering".

- Bus Mastering is a capability where a device with its own bus controller can control its own transfer of data to another device, freeing up the CPU for other tasks.

12. Explain the factors you would base on while selecting a network interface card to purchase.

- The type of network (Ethernet, Token Ring), the type of media (twistedpair, coaxial, fiberoptic, wireless), and the type of system bus (PCI is faster than ISA).

D. POWER DEVICES

1. Give different DC voltage level produced by AT power supply with their use. Specify also the colors of the power supply wires with their voltages.

- +12V (Yellow): Disk drives, fans, cooling devices, system slots.

- 12V (Blue): Some serial ports circuits, earlier PROM.

- +5V (Red): Motherboard, earlier CPUs, many motherboard components.

- 5V (White): ISA bus cards, early PROM.

- 0V (Black): Ground to complete circuit.

2. What are the main differences between AT and ATX power supplies?

- AT has two 6pin motherboard connectors (P8/P9); ATX has a single 20pin connector (P1).

- AT fan pulls air in from the rear and blows it on the motherboard; ATX fan pulls air through the case from the front and exhausts out the rear.

- ATX also produces +3.3V.

3. What is an offline UPS?

- An offline UPS remains idle until a power failure occurs, then switches from utility power to its own power source almost instantaneously.

4. What is an Online UPS?

- An online UPS continuously powers the protected load from its reserves (batteries) while simultaneously replenishing them from AC power. It protects against all common power problems and is also known as a power conditioner.

E. COMPUTER HARDWARE COMPONENTS

1. Which basic operating system function receives data from the keyboard?

a) Input.

b) Processing.

c) Output.

d) Storage.

Answer: a) Input.

2. In MSDOS, which of the following actions will result in a warm boot?

a) Replacing the CPU.

b) Pressing CTRL+ALT+DEL.

c) Powering on the system.

d) Regaining power after a power outage.

Answer: b) Pressing CTRL+ALT+DEL.

3. Which type of motherboard supports 3.3 volts?

a) AT.

b) ATX.

Answer: b) ATX.

4. Which types of RAM is used for cache memory?

a) SRAM (Static Random Access Memory).

b) DRAM (Dynamic Random Access Memory).

Answer: a) SRAM (Static Random Access Memory).

5. Which type of storage devices uses a laser to record information?

a) Hard disk.

b) Floppy disk.

c) CDRW.

d) Tape drive.

Answer: c) CDRW.

6. Which computer component can contain a dangerous amount of voltage?

a) Motherboard.

b) Monitor.

c) Hard drive.

d) CD ROM.

Answer: b) Monitor.

7. What is the function of the computer power supply?

a) Convert DC to AC.

b) Convert AC to DC.

c) Eliminate spikes in the electricity.

d) Eliminates brownout in the electricity.

Answer: b) Convert AC to DC.

8. Match the power supply to the voltage to the standard wire color.

| Power supply voltage | Wire color |

|||

| +12vdc               | Yellow     |

| 12vdc               | Blue       |

| +5vdc                | Red        |

| 5vdc                | White      |

| +3.5vdc              | Orange     |

| Ground               | Black      |

9. What is the result of installing the LED connectors to the motherboard incorrectly?

a) The motherboard will short out.

b) LED status light will still work.

c) LED status light will not work.

d) The computer will beep constantly when powered on.

Answer: c) LED status light will not work.

10. How are the hard drive and CDROM configured to be master and slave?

a) CMOS setup utility.

b) During partitioning.

c) Set jumpers.

d) During formatting.

Answer: c) Set jumpers.

11. How many volts do disk drive motors require?

a) 3.3 volts.

b) 5 volts.

c) 12 volts.

d) 120 volts.

Answer: c) 12 volts.

12. Which colored wire from a computer's power supply is the ground?

a) Red.

b) Black.

c) Yellow.

d) Blue.

Answer: b) Black.

13. What is the purpose of the heat sink?

a) To cool the computer processor.

b) To set the processor voltage.

c) To set the processor speed.

d) To ground the processor.

Answer: a) To cool the computer processor.

14. Some keys which allow you to access the CMOS Setup screen are DEL, CTRL+ALT+DELETE and F2.

a) TRUE

b) FALSE

Answer: a) TRUE.

15. Discuss on how the CDROM reads data from the CD.

- A laser beam is aimed at the disk from the bottom. The reflection from pits (less light, logic 0) and lands (more light, logic 1) is interpreted by a photo detector.

16. Explain how the Hard disk drive writes and reads data to and from platters.

- Write: A current pulse to the magnetization coil in the read/write head changes magnetization direction in the area under the head.

- Read: The head senses the magnetization state; movement changes the magnetic field, inducing a voltage whose polarity gives the stored bit.

17. Explain the steps performed during the computer system power on self test (POST).

- CPU checks itself, then the system timer. Checks RAM by writing and reading data. Sends messages to monitor or beep codes if errors are found. If no errors, sends one beep and begins loading the OS.

18. How can you identify an ATX motherboard?

- Has integrated I/O ports and PS/2 mouse connectors, CPU and RAM located next to the power supply, supports 3.3 volts operation, expansion slots parallel to the short side of the board.

19. Give the different types of expansion slots available on the Computer Motherboard.

- ISA

- EISA

- PC

- AGP.

20. Discuss why you would choose to build your own PC rather than buy a readymade PC. You should list at least seven reasons to build your own PC and three reasons to buy a readymade PC.

- Reasons to build own PC (7): Learning experience, exact component selection, potentially lower cost, better warranty control, customization, easier future upgrades, pride of building.

- Reasons to buy readymade PC (3): Convenience, single warranty/support point, often includes bundled software.

21. What are the reasons for partitioning a hard disk before installing an operating system in the computer?

- To organize data, to install multiple operating systems, to use different file systems (e.g., FAT32 and NTFS), to improve efficiency (smaller clusters), to isolate system files from user data.

22. Why is it necessary to format a drive before using a storage device?

- Formatting prepares a storage device for use by creating a file system structure (e.g., FAT, NTFS) that the operating system needs to read and write files. On a hard drive, it follows partitioning.

23. What are the functions of the standoffs that sit on the motherboard?

- Nonconductive pegs that prevent the motherboard from touching the metal case, avoiding short circuits.

24. Give the causes of the processor failure and how to troubleshoot this failure.

- Causes: Incorrect voltage setting, overheating (fan failure), physical damage (bent/broken pins), electrostatic discharge.

- Troubleshooting: Check voltage settings, ensure heat sink/fan works, inspect for physical damage, replace CPU.

25. What are the functions of the data bus on the motherboard? What are some of the bus sizes that you can have on a motherboard?

- Functions of the data bus: Carries data information bidirectionally between the CPU and other components.

- Bus sizes: 8bit, 16bit, 32bit, 64bit.

26. Give the steps of Hard disk drive configuration for installing in your personal computer.

- Set jumpers (Master/Slave/Cable Select).

- Physically mount the drive in a bay.

- Connect the appropriate IDE ribbon cable (40pin, 80conductor for modern drives).

- Connect a power cable from the power supply.

- Partition the drive (using FDISK or similar).

- Format the drive (using FORMAT command or OS tools).

F. OPERATING SYSTEMS

1. Define the term "Operating system".

- An operating system is a software program that controls thousands of operations, provides an interface between the user and the computer, and runs applications. It is in charge of running the computer.

2. Give and explain three components of any operating system.

- Kernel: Core piece of code loaded into memory at boot; manages hardware devices, memory allocation, system processes.

- User Interface (UI): The component the user interacts with; acts as a bridge between the user and the kernel.

- File System: Determines how files are named and placed on storage devices; defines how data is physically arranged on media.

3. Give and explain the Functions of an Operating system.

- File and folder management.

- Management of applications.

- Support for builtin utility programs.

- Control of computer hardware.

4. Explain the following terms as related to operating system:

- Multiuser: Two or more users running programs and sharing peripheral devices at the same time.

- Multitasking: A computer's capability to run multiple applications at the same time.

- Multiprocessing: Allows a computer to have two or more CPUs that programs share.

- Multithreading: The capability of a program to be broken into smaller parts that can be loaded as needed, allowing individual programs to be multitasked.

5. What are the three distinct sections that make up the disk operating system? Explain the function of each section.

- Boot files: Used during the boot process (startup).

- File management files: Enable the system to manage data in a system of files and folders.

- Utility files: Enable the user to manage system resources, troubleshoot, and configure settings.

6. Give and explain the common attributes for DOS files.

- Hidden File: Not visible with a normal file search.

- Read Only: Can be opened and read, but not written or modified.

- Archive: Indicates a backup copy of a file.

- System File: A file needed by DOS for a successful bootup.